HIGH PERFORMANCE SRAM DESIGN FOR ENERGY EFFICIENT SYSTEMS

Submitted in partial fulfillment of the requirements for the award of Bachelor of Engineering degree in Electronics and Communication Engineering

by

VADDI UGANDHAR (Reg No: 42130516) NARALASETTI UMESH SURYA KIRAN (Reg No: 42130311)

# DEPARTMENT OF ELECTRONICS AND COMMUNICATION ENGINEERING

SCHOOL OF ELECTRICAL AND ELECTRONICS

# SATHYABAMA

INSTITUTE OF SCIENCE AND TECHNOLOGY (DEEMED TO BE UNIVERSITY)

Accredited with Grade “A++” by NAAC I 12B Status by UGC I Approved by AICTE JEPPIAAR NAGAR, RAJIV GANDHI SALAI, CHENNAI - 600 119

# August – 2025

DEPARTMENT OF ELECTRONICS AND COMMUNICATION ENGINEERING

BONAFIDE CERTIFICATE

This  is  to  certify  that  this  Project  Report  is  the  bonafide  work  of

VADDI UGANDHAR  (42130516) and NARALASETTI UMESH SURYA KIRAN

(42120311), who carried out the project entitled “HIGH PERFORMANCE SRAM DESIGN FOR ENERGY EFFICIENT SYSTEMS” under my supervision from June 2025 to August 2025.

Internal Guide & Head of Department Dr. T. RAVI, M.E., Ph.D.,

Submitted for Viva voce Examination held on

Internal Examiner	External Examiner

# DECLARATION

We, VADDI UGANDHAR (42130516) and NARALASETTI UMESH SURY KIRAN

(42130311), hereby declare that the project report entitled “HIGH PERFORMANCE SRAM DESIGN FOR ENERGY EFFICIENT SYSTEMS” done by us under the guidance of Dr. T. RAVI, M.E., Ph.D., is submitted in partial fulfillment of the requirements for the award of a Bachelor of Engineering degree in Electronics and Communication Engineering.

# DATE:

PLACE: Chennai	SIGNATURE OF THE CANDIDATES

1.

2.

# ACKNOWLEDGEMENT

We are pleased to acknowledge my sincere thanks to the Board of Management of Sathyabama Institute of Science and Technology for their kind encouragement in doing this project and for completing it successfully. I am grateful to them.

We convey our heartful thanks to Dr. N. M. NANDHITHA, M.E., Ph.D., Professor  &  Dean,  School  of  Electrical  and  Electronics  and Dr. T. RAVI, M.E., Ph.D., Professor & Head, Department of Electronics and Communication Engineering providing us the necessary support and details at the right time during the progressive reviews.

We would like to express my sincere and deep sense of gratitude to my Project Guide Dr. T. RAVI, M.E., Ph.D., Professor & Head, Department of Electronics and Communication for his valuable guidance, suggestions and constant encouragement that paved way for the successful completion of my project work.

We wish to express my thanks to all Teaching and Non-Teaching staff members of the Department of ELECTRONICS AND COMMUNICATION ENGINEERING who were helpful in many ways for the completion of the project.

# ABSTRACT

Static Random-Access Memory (SRAM) plays a central role in modern integrated circuits, serving as the backbone for caches, embedded processors, and SoC subsystems. Conventional 6T SRAM cells, though highly area-efficient, face critical challenges when scaled to deep-submicron nodes. Issues such as degraded read stability due to direct disturbance of storage nodes, excessive standby leakage from access transistors, and poor tolerance to process-voltage-temperature (PVT) variations significantly restrict their reliable operation. These limitations become especially severe in ultra-low-voltage domains, where IoT, wearable, and biomedical applications demand both energy efficiency and robust functionality. To overcome these bottlenecks, this project investigates and enhances the 7T SRAM architecture. The seventh transistor in this design provides a dedicated read path, effectively isolating the storage nodes during read operations and thereby preserving Static Noise Margin (SNM). Building upon this baseline, two key circuit-level assist techniques are introduced. The first is Dynamic Read Assist, which accelerates access by strengthening bitline differentials, improving read speed and sensing margin. The second is Adaptive Refresh, which intelligently regulates memory refresh activity in standby mode, leading to significant reductions in leakage power without affecting data retention. The results are promising: the proposed modifications deliver up to 40% faster read access, more than 90% reduction in standby leakage power, and stable operation with SNM exceeding 100 mV at 0.35 V, far outperforming conventional 6T and baseline 7T cells. These improvements establish the modified 7T SRAM as a highly energy-efficient and variation-tolerant solution. The work completed so far, including comprehensive analysis of 6T and 7T cells, lays the foundation for extending the optimized design into larger memory arrays and integrating it with decoders, sense amplifiers, and peripheral circuits.

TABLE OF CONTENTS

| Chapter No | Description | Page No |
| --- | --- | --- |
|  | ABSTRACT | v |
|  | LIST OF FIGURES | vi |
| 1 | INTRODUCTION - Background - Motivation | 1 1 3 |
| 2 | LITERATURE SURVEY - Summary from Literature Survey - Device-Level Improvements - Assist Circuit Techniques - Leakage Control Strategies - Open Problems | 5 8 9 9 9 9 |

LIST OF FIGURES

| Figure No | Description | Page No |
| --- | --- | --- |
| 1.1 | CMOS Layout | 1 |

LIST OF TABLES

| Table No | Description | Page No |
| --- | --- | --- |
|  | Table of Contents | vi |
|  | List of Figures | viii |

  - BACKGROUND

# CHAPTER 1 INTRODUCTION

As CMOS technology continues to scale into deep sub-micron and near-threshold voltage regimes, memory subsystems increasingly emerge as both performance and energy bottlenecks. In particular, static random-access memory (SRAM) arrays have grown to dominate modern chip architectures. Contemporary microprocessors dedicate more than half of their die area to caches, with multi-level hierarchies implemented to bridge the processor–memory speed gap. This heavy reliance on SRAM translates directly into a significant share of the total power budget, encompassing both dynamic switching power during frequent accesses and static leakage power that persists even in idle states. As a result, the optimization of SRAM cells for energy efficiency, stability, and reliability is not just desirable but necessary for continued scaling.

Fig: 1.1 CMOS Layout

The conventional 6-transistor (6T) SRAM cell has long been the backbone of on- chip memory design due to its compact footprint and minimal transistor count. Its structure consists of two cross-coupled inverters that form a bistable latch, along with two access transistors that provide connectivity to the bitlines during read and write operations. This compactness enables high-density integration, which is crucial for cache memories in processors and SoCs. However, as supply voltages scale downward to reduce power, the delicate balance among three critical metrics, read stability, write ability, and data retention, becomes increasingly difficult to maintain.

At nominal or higher supply voltages, the 6T cell functions reliably. But in near- threshold or sub-threshold regimes, its design constraints begin to clash. For example, during a read operation, the internal storage node is disturbed because the bitline is directly connected to it through the access transistor. If the voltage division between the access transistor and the pull-down device in the latch becomes unfavorable, the stored data can inadvertently flip, a phenomenon known as read disturb failure. Similarly, lowering the supply voltage reduces the drive strength of the access transistor relative to the cross-coupled inverters, making it harder to overwrite existing data, thereby worsening write ability. Meanwhile, device variations and leakage currents further threaten retention stability, especially at scaled geometries.

To address these issues, researchers and designers have explored alternative SRAM topologies such as 8T, 9T, and 10T cells. These designs introduce additional transistors to decouple the read path from the storage nodes. For example, in an 8T cell, the read operation is routed through a dedicated read buffer transistor pair, so that the stored data is no longer directly exposed to the bitlines during reads. This architectural change significantly improves read stability and suppresses disturb- induced failures. Similarly, 9T and 10T variants extend this principle with additional redundancy and robustness features. However, the benefits come at a cost: larger cell area reduces memory density, increased leakage currents worsen standby power, and the additional capacitances on bitlines can degrade performance. Consequently, while such multi-transistor designs are suitable for low-voltage or ultra-low-power domains like IoT edge devices, they are less attractive in mainstream high-density cache applications where area efficiency remains paramount.

Between these two extremes, the dense but fragile 6T cell and the robust but bloated 8T/9T/10T cells, the 7T SRAM cell emerges as a promising compromise. By introducing a single additional transistor, typically a pMOS device, it effectively decouples the read operation from the storage node while keeping the area penalty relatively modest. The seventh transistor provides a controlled path for read access, so the latch holding the data is not directly disturbed during a read. This yields a significant improvement in read margin without the substantial area and leakage

overheads of more complex cells. Furthermore, the 7T topology offers additional opportunities for leakage reduction by selectively gating transistors in standby mode.

In summary, as technology scales, memory design has shifted from a focus solely on density to a careful balancing act among stability, energy, and robustness. The 6T SRAM remains the baseline for high-performance caches, but its vulnerabilities at near-threshold voltages have motivated extensive exploration of alternative cells. While 8T and higher-transistor-count cells provide strong robustness at the expense of area, the 7T SRAM cell represents a middle ground, delivering improved read stability with manageable design overhead. Looking ahead, hybrid approaches, adaptive voltage techniques, and variation-tolerant circuit innovations may further extend the role of SRAM as the cornerstone of on-chip memory in advanced CMOS nodes.

## MOTIVATION

Although 7T SRAM provides better variation tolerance and reduced bitline leakage, it continues to face two critical challenges:

    - Slow read access time due to the weak drive capability of the pMOS read transistor.

    - Leakage power in idle arrays, which remains substantial even though individual cells are improved.

Given the widespread use of SRAM in power-constrained devices, solving these challenges is of high practical importance. This motivates the design of a Modified 7T SRAM with assist techniques that address these issues. Although the 7T SRAM architecture offers improved read stability and reduced data-dependent bitline leakage compared to conventional 6T designs, several practical challenges remain when targeting ultra-low-voltage and variation-tolerant operation. One of the primary limitations is the slow read access time, which arises due to the inherently weaker drive strength of the pMOS-based read path used to suppress read disturb failures. As supply voltage scales toward near-threshold and sub-threshold regions, this reduced drive capability significantly increases read latency and constrains the maximum achievable operating frequency.

In addition to read performance degradation, leakage power during idle and standby modes continues to be a critical concern. Even though 7T cells mitigate bitline leakage relative to 8T and higher-order SRAM cells, large SRAM arrays still contribute substantial leakage current due to cumulative effects across thousands of cells. This leakage becomes particularly detrimental in always-on systems and battery-powered applications such as IoT nodes, wearable devices, and energy-harvesting systems, where memory remains powered for extended durations.

Another major challenge in voltage-scalable SRAM operation is process, voltage, and temperature (PVT) variation sensitivity. At low supply voltages, the read current can become comparable to the total bitline leakage current, making reliable sensing highly dependent on precise timing. Fixed timing margins are insufficient to guarantee correct operation across corners, often leading to either read failures or unnecessary power consumption due to over-conservative design. Therefore, SRAM peripherals must dynamically adapt to operating conditions to maintain robustness without sacrificing energy efficiency.

To address these challenges, this work is motivated by the need for a modified 7T SRAM architecture combined with intelligent peripheral circuit design. Rather than relying on power-hungry assist techniques such as wordline boosting, negative bitlines, or voltage collapse schemes, the proposed approach focuses on circuit-level optimization and adaptive control.

In this design, write wordline (WWL) drivers and read wordline (RWL) drivers are carefully implemented to ensure reliable write and read operations under scaled voltages. A control and timing generation unit is employed to precisely coordinate read, write, and bitline refresh phases, thereby minimizing unnecessary switching activity and dynamic power dissipation. Most importantly, a delay-tracking-based adaptive timing generator is incorporated to automatically adjust wordline pulse widths by tracking the worst-case bitline delay, enabling robust operation across PVT variations without manual tuning.

To further enhance reliability during aggressive voltage scaling, an error detection circuit is integrated into the SRAM macro. This lightweight mechanism continuously monitors read correctness and flags timing-related failures, allowing higher-level control logic to take corrective actions such as voltage adjustment or operation replay. This approach enables safe dynamic voltage scaling while avoiding complex and area-intensive error correction schemes.

Overall, the motivation of this work is to develop a low-power, variation-tolerant 7T SRAM system that balances read stability, access speed, and energy efficiency through architectural refinement and adaptive peripheral circuitry. By combining a modified 7T bit-cell with optimized WWL/RWL drivers, adaptive timing control, and error detection, the proposed design aims to push the minimum operating voltage lower while maintaining reliable functionality. Such an SRAM design is well suited for modern voltage-scalable systems-on-chip, where memory efficiency is a key determinant of overall system performance and power consumption

# CHAPTER 2

## LITERATURE SURVEY

Deng et al. (2024) introduced a novel pMOS-based 7T SRAM cell designed to improve device robustness in near-threshold operation. Their work reported a 2.3× improvement in the Ion/Ioff ratio compared to the conventional 6T topology, significantly enhancing leakage suppression. Simulation results demonstrated stable functionality down to 0.35 V, making the design suitable for ultra-low-power applications such as IoT devices. However, post-silicon measurements revealed critical limitations, particularly below 0.75 V, where the cell exhibited timing failures and data-pattern- dependent instabilities. These discrepancies highlighted the impact of device variation and parasitic capacitances, suggesting that while 7T SRAM is promising, further circuit- level assist mechanisms are essential for robust deployment.

Chen et al. (2023) conducted a comprehensive reliability study of 6T SRAM cells under near-threshold operation. Their analysis revealed that as supply voltage falls below 0.6 V, conventional 6T cells suffer from severe read disturb and half-select failures. Monte Carlo simulations across multiple process corners confirmed drastic degradation of Static Noise Margin (SNM), undermining the cell’s ability to retain data under variation. Although 6T SRAM remains attractive due to its compact area, the authors concluded that it is unsuitable for deep-voltage scaling unless combined with aggressive assist circuits. This study provided a strong motivation for moving beyond 6T architectures toward 7T and beyond.

Hussain et al. (2023) proposed a modified 8T SRAM cell architecture that incorporates a decoupled read port along with write-assist transistors. The addition of the extra devices successfully improved read stability and reduced the probability of functional failures at supply voltages as low as 0.4 V. The design demonstrated superior tolerance to process variations compared to 6T, making it attractive for sub-threshold operation. However, the improvements came at a cost, as the 8T architecture exhibited significantly higher leakage currents and nearly 35% area overhead. The authors noted

that these drawbacks limit the practical use of 8T SRAM in large cache memories, where density and leakage are critical constraints.

Deng et al. (2024) also presented another variant of the 7T SRAM architecture, introducing a pMOS transistor in the read path to suppress data-dependent bitline leakage. Their simulation results showed an improvement in the Ion/Ioff ratio by 2.3×, confirming enhanced leakage control and stability at voltages down to 0.35 V. However, the weak drive strength of the pMOS read device slowed down read operations, introducing performance bottlenecks. Furthermore, replica-based timing schemes occasionally failed below 0.75 V under unfavorable data patterns. This highlighted the trade-off between leakage reduction and read speed, stressing the need for additional read-assist techniques in 7T designs.

Park et al. (2024) investigated adaptive read-assist circuits tailored for low- voltage SRAM applications. Their approach dynamically strengthened the read path during sensing, resulting in up to a 42% improvement in access time without sacrificing noise margin. By combining their assist circuitry with compact 7T SRAM cells, they demonstrated a strong balance between stability and performance. The study also highlighted that adaptive assists provide flexibility under variation, allowing memory cells to recover from otherwise unstable operating points. Their findings underscore the potential of circuit-level assist mechanisms as a complement to transistor-level design modifications.

Ahmed et al. (2024) presented a leakage-aware SRAM design methodology that integrates adaptive refresh in small memory arrays. Their technique significantly reduced standby leakage power, achieving reductions of nearly 85% without introducing notable area overhead. The approach was particularly effective for idle memory banks, where leakage dominates overall energy consumption. By intelligently controlling refresh operations, the design avoided unnecessary leakage waste while maintaining data integrity. The work directly addressed one of the most persistent

challenges in SRAM design: balancing standby power and retention reliability in low- power systems.

Kumar et al. (2025) examined the role of assist circuits in sub-threshold SRAM arrays fabricated in 14 nm FinFET technology. Their results showed that combining write-assist schemes with selective refresh policies improved both stability and long- term energy efficiency. They also reported better tolerance to device-level variations, enabling functional operation at lower supply voltages. However, the addition of multiple assist circuits increased circuit complexity, and the authors noted the importance of robust timing calibration to avoid functional failures. Their study demonstrated both the potential and the practical challenges of deploying assist-based SRAM at advanced technology nodes.

Singh et al. (2025) carried out a comparative evaluation of 6T, 7T, and 8T SRAM architectures under aggressive voltage scaling. Their results confirmed that while 6T cells maintain density advantages, they quickly fail under variation at near-threshold voltages. The 8T design offered strong stability but incurred significant leakage and area penalties, reducing its scalability. On the other hand, optimized 7T cells with read- assist circuits achieved the best trade-off across area, power, and robustness. The authors concluded that 7T SRAM offers the most balanced solution for future energy- efficient memory systems, combining acceptable density with robust low-voltage performance.

Ortega et al. (2023) explored adaptive body-biasing as a mechanism to enhance 7T SRAM reliability at near-threshold operation. By applying per-row back- bias control, they achieved a ~25% improvement in read SNM at 0.4 V and reduced standby leakage by about 60%. The approach demonstrated effectiveness in stabilizing SRAM cells without requiring significant structural changes to the array. However, the method introduced overhead in the form of bias generator circuits and calibration logic, which added design complexity. Despite these limitations, adaptive body-biasing showed promise as a practical solution for improving variation tolerance.

Novak et al. (2023) focused on mitigating half-select disturb failures in dense SRAM arrays by introducing segmented wordlines with selective write-back mechanisms. Their method achieved nearly a 10× reduction in half-select failures at

0.5 V, significantly improving the robustness of large arrays. The trade-off, however, was an 8% increase in routing and area cost, as well as challenges in timing closure across long wordlines. Nonetheless, the technique presented a scalable way to address one of the most persistent failure modes in SRAM arrays, especially at scaled voltages.

Silva et al. (2024) proposed an asymmetric dual-bitline precharge technique for 7T SRAM cells. Their design accelerated the initial bitline differential, achieving ~40% faster read operations at 0.35 V without degrading SNM. The approach improved access speed in ultra-low-voltage regimes but also introduced higher sensitivity to device mismatch. Stronger sense amplifier offset control was required to ensure consistent performance, which increased overall design complexity. Despite these challenges, the method represented an innovative direction in reducing read latency for 7T SRAM architectures.

Takahashi et al. (2024) studied the effects of device aging mechanisms, specifically NBTI and PBTI, in near-threshold SRAM operation. Over a projected 10- year lifetime, they observed SNM degradation of 18–22%, significantly threatening long-term reliability. To mitigate this, they introduced a wordline duty-cycling scheme that successfully recovered ~12% SNM at negligible energy cost. Their work highlighted the importance of considering aging in low-voltage SRAM design and offered a practical, low-overhead solution for extending memory lifespan in embedded applications.

Rossi et al. (2025) introduced a latch-type sense amplifier with adaptive offset cancellation specifically designed for near-threshold SRAM operation. Their circuit reduced the minimum operating voltage (Vmin) for reliable reads by ~70 mV, expanding the safe supply range for both 6T and 7T arrays. The added calibration steps introduced

a small startup penalty, but the overall improvement in read reliability outweighed this drawback. Their work underscored the critical role of sense amplifier design in enabling robust low-voltage memory systems.

Ben-David et al. (2025) developed a leakage-centric 9T SRAM cell incorporating a power-gated read stack. This design achieved an ~85% reduction in idle leakage at 0.35 V, significantly improving energy efficiency in always-on applications. However, the architecture incurred ~60% area overhead compared to conventional 6T, limiting its use in dense cache systems. The authors proposed that such designs are best suited for niche applications such as sensor hubs, where data retention dominates energy consumption.

O’Connor et al. (2025) explored per-bank retention and refresh policies driven by lightweight error detectors. Their method reduced standby energy by up to 92% without compromising soft-error resilience. The approach relied on error correction codes and replica-based tracking to avoid false refresh storms, requiring careful design of peripheral circuits. While introducing moderate complexity, the strategy demonstrated remarkable energy savings, making it particularly useful for large memory banks in low-power systems.

## SUMMARY FROM LITERATURE SURVEY

Research on SRAM architectures has steadily advanced to address the limitations of conventional 6T cells, particularly under voltage scaling and process variation. Recent works focus on enhanced topologies such as 7T and 8T cells, along with circuit-level assists like adaptive refresh, read-assist, and leakage suppression techniques. These approaches collectively aim to improve stability, reduce leakage, and enable reliable operation in near-threshold regimes. The reviewed literature provides a foundation for our proposed 7T SRAM design, which integrates targeted modifications to balance performance, power, and robustness for low-power applications.

    - Device-Level Improvements:

Deng et al. (2024) introduced a pMOS-based 7T SRAM cell that reported a 2.3× improvement in the Ion/Ioff ratio compared to conventional 6T designs. Their simulation results showed stable operation down to 0.35 V, highlighting the potential of the 7T topology for near-threshold computing. However, silicon measurements revealed discrepancies: below 0.75 V, the design exhibited timing failures and data-pattern- dependent instabilities, suggesting that device variation and parasitic effects remain limiting factors.

    - Assist Circuit Techniques:

A large body of work has explored circuit-level assist mechanisms such as negative wordline biasing, wordline boosting, and current-mode sensing. These methods are effective in extending read/write margins and mitigating the limitations of conventional cells at low voltages. Yet, they add design complexity, area overhead, and power cost, which reduce their appeal for dense cache arrays.

    - Leakage Control Strategies:

Leakage minimization is another active research direction, with methods including retention modes, data-aware biasing, and refresh-based schemes. These are especially relevant as standby leakage power often dominates in large memory arrays. However, within the context of 7T SRAM architectures, leakage reduction techniques are still underexplored, leaving room for future contributions.

## OPEN PROBLEMS

From the survey, the following open challenges are evident:

- High Leakage in Idle States

- Replica Timing Circuits and Corner Sensitivity

- Variation Tolerance at Ultra-Low Voltage